GOAL OF THE PROJECT

Traffic light is the common things that exist in this world since the development of car and roadside exist. This machine helps us human in monitoring the roadside user to walk or drive at the road much smooth and safe. This technology has the probability to reduce car accident at roads. The basic work principle in traffic light is there is three light that indicates as a signal for us which is green for drive or walk, yellow as a signal that the light will turn red and lastly red means we must stop. The number of traffic lights depend on the type of roadside. If it was four ways roadside, there should be four while three ways should be three. This project shows how to implement basic DLD component in creating three ways traffic light. In this project, there is two main DLD component which cascaded together to get the three ways traffic light system. The first component is counter and the second one is decoder. Basically, the output for the counter is used as the input of decoder and decoder will generated light for the output

DESIGN PROCESS

- COUNTER

Asynchronous counter used in this project. There is four T flip flop is used and each of the flip flop has HIGH as the input to make sure the output work in toggle mode.

The first flip flop is clocked by external clock while the other flip flop clocked by the output of previous flip flop. The modulus of the counter should be 16 but this project only used 15 as the number of unique states. This is because the tempo or period for one cycle for this project only 15 second which every traffic light has 4 second to be operate as green light and 1 second as yellow light. While the one of the traffic light operate, the others light up the red light to show that the way is on used for the one way that being operated. To make the modulus is 15 rather than 16, the (1111) output decode with NAND gate and the output of the gate is connected to CLR to clear all the flip flop output to (0000). Positive edge triggered is used, so the next flip flop clock is clocked by the output of the previous flip flop which is Q bar to make the counts is count up.

- DECODER

The output of the counter is used as input of the decoder. The output has 4 binary digits, so the decoder should be 4 to 16 line decoder. However, we only have 15 output for the decoder makes it 4 to 15 line decoder. This is because the last output for counter (1111) not used so the number of states become 15 (in the KMAP, the 1111 act as don’t care). The work principle of the decoder is when a unique state used in the input, the output for the decoder will start to operate based on the truth table below. For example, when the input ABCD is 0000, the G1 (first green traffic light) will light on with R2(second red traffic light) and R3(third red traffic light). This indicate that the first way can go through while the other must stop and wait for their time to their green light to light up. This will keep on until 4 second end and at five second, the first traffic light will light up the yellow one to gave warning that it will light up the red one soon. At 6 second, the first one will turn red and second one will turn their green light(G2) to HIGH and the third one remains the same. This thing will keep going like when the first one light up the G1, the difference is this time G2 light up. After the second one turn Y2 HIGH and then R2 will turn on, the third one will operate in green light mode. This this also the same as the cycle before but the third one will turn green until G3 turn off and Y3 turn on in 1110 states. Then its start again like when the first one G1 light up.

DETAILED DESIGN